Department of Electrical & Computer Engineering

First Semester 2025/2026

HW Design Lab

Project#1 – Design Verification (APB FIFO Verification using UVM)

Instructors: Abdellatif Abu-Issa and Ayman Hroub

Teaching Assistants: Raha Zabadi, Jehad Halahala, and Tala Abahra

## Design Overview

Your task is to verify a buggy RTL design of an APB-based FIFO using the UVM methodology. You will build a complete UVM testbench, including an APB agent, register model, scoreboard, and coverage collection.

## Design Overview

The DUT is a parameterized synchronous FIFO accessed through an APB3 slave interface. Software can push bytes into the FIFO via the DATA register and pop them by reading the same register. The design includes programmable thresholds, control bits, and sticky status flags.

### Main Parameters

• WIDTH = 8 bits
• DEPTH = 16 entries

### APB Interface

PSEL, PENABLE, PWRITE, PADDR[7:0], PWDATA[31:0], PRDATA[31:0], PREADY, PSLVERR

### Register Map

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | CTRL | RW | Bit[0] EN, Bit[1] CLR, Bit[2] DROP_ON_FULL |
| 0x04 | THRESH | RW | Bit[7:0] ALMOST_FULL_TH, Bit[15:8] ALMOST_EMPTY_TH |
| 0x08 | STATUS | RO/RC | Empty/Full/Almost/Overflow/Underflow/Count |
| 0x0C | DATA | RW | Push or pop one byte |

## Functional Rules

1. FIFO operations depend on CTRL.EN.
2. Writing CTRL.CLR clears FIFO and sticky flags.
3. DATA writes push data when not full; reads pop data when not empty.
4. Overflow/Underflow flags are sticky until cleared.
5. DROP_ON_FULL defines whether to drop or error on full.
6. ALMOST_FULL and ALMOST_EMPTY depend on programmable thresholds.

## 3. What You Must Build (UVM Testbench)

• APB master agent (driver, sequencer, monitor)
• Register model (UVM_REG)
• Scoreboard with reference FIFO model
• Functional coverage
• Directed and random sequences covering all operations and errors

## Deliverables

• UVM testbench source code
• Coverage and report summary
• Short bug report (observed failures and reasoning)

## 4. Buggy RTL (apb_sync_fifo.sv)

The following RTL contains several functional bugs intentionally added for verification purposes. You must identify them through simulation and coverage-driven verification. Do NOT modify the RTL.

//------------------------------------------------------------------------------

// APB3-accessible synchronous FIFO (WIDTH=8, DEPTH=16) — intentionally buggy

//------------------------------------------------------------------------------

module apb_sync_fifo #(

parameter int WIDTH = 8,

parameter int DEPTH = 16,

localparam int AW    = $clog2(DEPTH)

)(

input  logic        PCLK,

input  logic        PRESETn,

input  logic        PSEL,

input  logic        PENABLE,

input  logic        PWRITE,

input  logic [7:0]  PADDR,

input  logic [31:0] PWDATA,

output logic [31:0] PRDATA,

output logic        PREADY,

output logic        PSLVERR

);

// ---- APB constants ----

localparam CTRL_O   = 8'h00;

localparam THRESH_O = 8'h04;

localparam STATUS_O = 8'h08;

localparam DATA_O   = 8'h0C;

// ---- registers ----

logic        en, clr, drop_on_full;

logic [7:0]  almost_full_th, almost_empty_th;

logic        empty, full, almost_full, almost_empty;

logic        overflow, underflow; // sticky

logic [AW:0] count;               // allows DEPTH value

// FIFO storage

logic [WIDTH-1:0] mem [0:DEPTH-1];

logic [AW-1:0] wptr, rptr;

// APB default

assign PREADY  = 1'b1;     // zero-wait state

assign PSLVERR = 1'b0;

// Async reset

always_ff @(negedge PRESETn or posedge PCLK) begin

if (!PRESETn) begin

en <= 1'b0;

clr <= 1'b0;

drop_on_full <= 1'b0;

almost_full_th <= DEPTH-1;

almost_empty_th <= 8'd1;

end else begin

if (PSEL && !PENABLE && PWRITE) begin // setup phase latching

unique case (PADDR)

CTRL_O: begin

en           <= PWDATA[0];

clr          <= PWDATA[1];   // self-clear handled below

drop_on_full <= PWDATA[2];

end

THRESH_O: begin

almost_full_th  <= PWDATA[7:0];

almost_empty_th <= PWDATA[15:8];

end

default: ;

endcase

end

// self-clear of CLR

if (clr) clr <= 1'b0;

if (PSEL && PENABLE && !PWRITE) begin

overflow <= 1'b0;

underflow <= 1'b0;

end

end

end

// STATUS read mux

always_comb begin

PRDATA = 32'd0;

unique case (PADDR)

STATUS_O: PRDATA = {18'd0, count[7:0], underflow, overflow, almost_empty, almost_full, full, empty};

DATA_O:   PRDATA = {24'd0, mem[rptr]};

CTRL_O:   PRDATA = {29'd0, drop_on_full, clr, en};

THRESH_O: PRDATA = {16'd0, almost_empty_th, almost_full_th};

default:  PRDATA = 32'd0;

endcase

end

// FIFO control

logic push, pop;

assign push = (PSEL && PENABLE && PWRITE && (PADDR==DATA_O));

assign pop  = (PSEL && PENABLE && !PWRITE && (PADDR==DATA_O));

// Count/flags

always_ff @(posedge PCLK or posedge PRESETn) begin

if (PRESETn) begin

count <= '0;

wptr  <= '0;

rptr  <= '0;

empty <= 1'b1;

full  <= 1'b0;

end else begin

if (clr) begin

count <= '0;

wptr  <= '0;

rptr  <= '0;

empty <= 1'b1;

full  <= 1'b0;

end else if (en) begin

// Push

if (push) begin

if (!full) begin

mem[wptr] <= PWDATA[7:0];

wptr <= wptr + 1'b1;

count <= count + 1'b1;

end else begin

if (drop_on_full) begin

// drop silently but set flag

overflow <= 1'b1;

end else begin

overflow <= 1'b1;

end

end

end

// Pop

if (pop) begin

if (!empty) begin

rptr <= rptr + 1'b1;

count <= count - 1'b1;

end else begin

underflow <= 1'b1;

end

end

// Update empty/full

empty <= (count == 0);

full  <= (count == DEPTH-1);

end

end

end

always_ff @(posedge PCLK or negedge PRESETn) begin

if (!PRESETn) begin

almost_full  <= 1'b0;

almost_empty <= 1'b1;

end else begin

almost_full  <= (count > almost_full_th);

almost_empty <= (count <= almost_empty_th);

end

end

endmodule

## 5. Grading Policy

• UVM testbench architecture – 30 pts
• Stimulus quality – 25 pts
• Scoreboard and checks – 20 pts
• Coverage– 10 pts
• Short report including Bugs report – 15 pts